# Local SOP: CDU Overhead Corrosion Monitoring

SYNTHETIC DEMO DATA - NOT AN OPERATING PROCEDURE

Facility: Kavach Demonstration Refinery | Unit: Crude Distillation Unit (CDU)

# 1. Purpose

Provide a demonstration knowledge source for reviewing corrosion observations in a crude distillation unit overhead system.

# 2. Roles

Panel operator: monitor overhead temperature, pH, and chloride trend.

Field operator: collect samples and verify chemical injection equipment.

Inspection engineer: complete visual checks and ultrasonic thickness follow-up.

# 3. Operating controls

Maintain wash-water injection at 5-7 percent of crude feed rate.

When sour-water chloride exceeds 50 ppm, increase wash-water injection by 10 percent and record the adjustment.

Maintain overhead accumulator boot-water pH between 5.5 and 6.5 through neutralizing-amine adjustment.

Maintain filming-amine dosage according to the approved chemical programme.

# 4. Inspection response

For localized pitting near a nozzle, schedule focused ultrasonic testing within 90 days.

For chloride-attack indicators, review the chloride trend and wash-water rate with Operations before the next integrity review.

Attach photographs and thickness readings to the integrity review pack.

# 5. Escalation

If overhead pH falls below 4.0, notify the shift supervisor and apply the approved emergency response plan.

This synthetic SOP does not authorize operational changes or replace site procedures.